M3508 Brushless DC Motor

M3508 直流无刷电机

User  Guide使用说明

v1.0     2025.10

Contents

Disclaimer                                                                                                                                                         2

Introduction                                                                                                                                                    2

In the Box                                                                                                                                                          2

Using the Product                                                                                                                                      2

Connecting to the MSC                                                                                                                                       2

Installation                                                                                                                                                                     3

Precautions                                                                                                                                                                   3

Performance and Parameters                                                                                                       4

Payload Characteristic Curves                                                                                                                      4

Characteristic Parameters                                                                                                                                5

Output Gear Parameters                                                                                                                                   5

目录

免责声明                                                                                        6

简介                                                                                              6

物品清单                                                                                        6

产品使用                                                                                        6

连接电调                                                                                                      6

安装                                                                                                             7

注意事项                                                                                                      7

性能与参数                                                                                     8

负载特性曲线                                                                                                8

特征参数                                                                                                      9

输出齿参数                                                                                                   9

# Disclaimer

By  using this  product, you  signify that you  have  read,  understand,  and  accept the terms and  conditions  of this  guideline  and  all  instructions  at  https://www.robomaster.com. THE  PRODUCT AND ALL  MATERIALS AND  CONTENT AVAILABLE THROUGH THE  PRODUCT ARE  PROVIDED  "AS  IS"  AND  ON  AN  "AS  AVAILABLE"  BASIS  WITHOUT  WARRANTY  OR CONDITION OF ANY KIND.

# Introduction

RoboMaster  M3508  Brushless  DC  Motor  (hereinafter  referred  to  as  the  "motor")  is designed for  high-performance  robots. When working with  RoboMaster  C620  Brushless DC  Motor  Speed  Controller  (hereinafter  referred  to  as  the  "MSC";  the  MSC  is  sold separately),  it  supports  sinusoidal  (sine-wave)  drive,  which  provides  higher  efficiency, better dynamic performance, and greater stability than traditional square-wave drive.

# In the Box

7-Pin Data Cable

Motor                                                                                      × 1

×  1

# Using the Product

## Connecting to the MSC

1.   Use the included 7-pin data cable to connect the 7-pin ports on the MSC and the motor.

2.  Connect the three-phase cable connectors of the  MSC and the  motor, ensuring that the wire colors match. DO NOT insert the connectors in the wrong direction.

2

## Installation

Refer  to  the  following  dimensions  to  select  the  mounting  holes  and  install  the  motor based on your needs.

-0.025

5.00 -0.035

13.00

2-M3     4   EQS

7.00   0.02

4-M3 Unit: mm

43.40

30.10 17.60

0

26.8 -0.06

35.00 42.00

4   EQS

8.30    1

2.00

## Precautions

|  | • The motor has M3 threaded holes with a depth of 4 mm. DO NOT use excessively long screws. Otherwise, the printed circuit board (PCB) may be damaged. • Make sure that the motor is securely installed. • Use the motor under the maximum winding temperature. Prolonged overheating can permanently damage the motor. • Avoid damage to the cables and wires. Otherwise, the motor may work improperly. • DO NOT touch the rotors to avoid injury. • DO NOT allow any foreign materials to come into contact with the rotors, as this may affect performance. • The motor will heat during high-power operation. Avoid direct contact to prevent burns. |
| --- | --- |

# Performance and Parameters

## Payload Characteristic Curves

Closed-Loop Speed Control Using C620 MSC

0.05   0.10   0.15   0.20   0.25   0.30   0.35

T (N·m)

η

n                                           I

P

n (rpm)

8000

7000

6000

5000

4000

3000

2000

1000

0

η (%)

80

70

60

50

40

30

20

10

0

0

P (W)

400

350

300

250

200

150

100

50

0

I (A)

16

14

12

10

8

6

4

2

0

Closed-Loop Current Control Using C620 MSC

n

I η

P

0     0.05   0.10   0.15  0.20  0.25  0.30  0.35

T (N·m)

n (rpm)

9000

8000

7000

6000

5000

4000

3000

2000

1000

0

P (W)

450

400

350

300

250

200

150

100

50

0

η (%)

90

80

70

60

50

40

30

20

10

0

I (A)

18

16

14

12

10

8

6

4

2

0

Operating Range Diagram

n (rpm)

10000

8000

6000

4000

2000

0

0.04        0.08        0.12 Continuous Operating Range

0.16

T (N·m)

125° C (257° F)

n-Rotational Speed, η-Efficiency, I-Current, P-Output Power, T-Torque

The above data were measured under laboratory conditions at an input voltage of 24 V, an ambient temperature of 25 °C (77 °F), and under normal cooling. They are provided for reference only. Adjust the motor running time based on the actual conditions such as operating temperature and cooling conditions.

| ## Characteristic Parameters |  |
| --- | --- |
| Motor 24 V 0.02 N·m/A 390 rpm/V 260 rpm/N·m 49 ms 0.194 Ω 0.097 mH 0° to 50° C (32° to 122° F) 125° C (257° F) 7 276 N 120 g 9400 rpm 0.6 A 9085 rpm 0.16 N·m Rated Voltage Torque Constant Rotational Speed Constant Speed-Torque Gradient Mechanical Time Constant Phase Resistance Phase Inductance Operating Temperature Maximum Winding Temperature Number of Pole Pairs Rated Dynamic Load Motor Weight Motor and C620 MSC at Rated Voltage* No-Load Speed No-Load Current Rated Rotational Speed Rated Torque (Maximum Continuous Torque) 10 A 83% 0.33 N·m Rated Current Maximum Efficiency Peak Torque |  |
| Peak Current 2.5 A |  |
| * All data in this table were measured under laboratory conditions. All current values represent the input current of the MSC, and the efficiency refers to the overall efficiency of the motor-MSC drive system. |  |
| ## Output Gear Parameters |  |
| Number of Teeth | 17 |
| Module | 0.6 |
| Pressure Angle | 20° |
| Pitch Diameter | 10.2 |
| Addendum Diameter | 11.38 |
| Dedendum Diameter | 8.68 |
| Profile Shift Coefficient | -0.01 |
| Tooth Span | 2 |
| Common Normal | -0.02 2.79-0.04 |

# 免责声明

使用本产品前，请您仔细阅读本文、访问 https://www.robomaster.com 阅读本产品相关的所有指引。使用本产品视为您已经阅读并接受本文与本产品相关的全部条款。

# 简介

RoboMaster  M3508  直流无刷电机（以下简称为“电机”） 专为高性能机器人设计，搭配RoboMaster C620 无刷电机调速器（以下简称为“电调”，电调需另外购买）可以实现正弦驱动，相比传统方波驱动具有更高的效率、机动性和稳定性。

# 物品清单

电机                                                  ×  1            7-Pin 数据线                                       ×  1

# 产品使用

## 连接电调

1.  使用包装内的 7-Pin 数据线连接电调和电机的 7-Pin 数据端口。

2.  连接电机的三相输入接头与电调的三相动力线接头，确保相同颜色的线对应连接。请勿强行反插接头。

2

## 安装

参考以下尺寸，按需选择孔位进行安装。

4-M34   EQS

-0.025

5.00 -0.035

13.00

2-M3     4   EQS

7.00   0.02

43.40

30.10 17.60

0

26.8 -0.06

35.00 42.00

单位：mm

8.30    1

2.00

## 注意事项

•  电机安装孔为 M3 螺纹孔，深度 4mm，请勿使用过长的螺丝，否则可能会顶到电机的PCB 板从而损坏电机。

•  确保电机安装稳固。

•  请在绕组最大允许温度范围内使用电机，避免长时间过热而损坏电机。

•  请避免损伤线材，以防电机运行异常。

• 使用时请勿触摸电机转子部分，避免割伤。

•  避免杂物进入转子内部，以免导致转子运行异常。

•  电机大功率运行时表面温度较高，请勿直接触碰以免烫伤。

# 性能与参数

## 负载特性曲线

使用 C620 电调做速度闭环控制

η

n                                           I

P

n (rpm)

8000

7000

6000

5000

4000

3000

2000

1000

0

η (%)

80

70

60

50

40

30

20

10

0

0

P (W)

400

350

300

250

200

150

100

50

0

I (A)

16

14

12

10

8

6

4

2

0

0.05   0.10   0.15   0.20   0.25   0.30   0.35

T (N·m)

使用 C620 电调做电流闭环控制

n

I η

P

0     0.05   0.10   0.15  0.20  0.25  0.30  0.35

T (N·m)

n (rpm)

9000

8000

7000

6000

5000

4000

3000

2000

1000

0

P (W)

450

400

350

300

250

200

150

100

50

0

η (%)

90

80

70

60

50

40

30

20

10

0

I (A)

18

16

14

12

10

8

6

4

2

0

工作范围图

n (rpm)

10000

8000

6000

4000

2000

0

0.04

持续工作范围

0.16

T (N·m)

125°C

0.08

0.12

n-转速，η-效率，I-电流，P-输出功率，T-扭矩

以上数据均在输入电压 24V、25℃ 室温、正常散热的实验环境下测得，  仅供参考。实际使用时，  请根据工作环境温度、散热条件等实际情况来控制电机运行时间。

## 特征参数

| 电机 额定电压 | 24 V |
| --- | --- |
| 转矩常数 | 0.02 N·m/A |
| 转速常数 | 390 rpm/V |
| 转速转矩梯度 | 260 rpm/N·m |
| 机械时间常数 | 49 ms |
| 相电阻 | 0.194 Ω |
| 相电感 | 0.097 mH |
| 使用环境温度 | 0℃至 50℃ |
| 绕组最高允许温度 | 125 ℃ |
| 极对数 | 7 |
| 额定动载荷 | 276 N |
| 电机重量 | 120 g |

电机搭配 C620  电调在额定电压下使用 *

| 空载转速 | 9400 rpm |
| --- | --- |
| 空载电流 | 0.6 A |
| 额定转速 | 9085 rpm |
| 额定转矩（最大连续转矩） | 0.16 N·m |
| 额定电流 | 10 A |
| 最大效率 | 83% |
| 峰转矩 | 0.33 N·m |

峰转电流                             2.5 A

* 本表格中的数据均在实验环境下测得。电流均为电调的输入端电流，效率为电机与电调组成的驱动系统整体效率。

| ## 输出齿参数 齿数 | 17 |
| --- | --- |
| 模数 | 0.6 |
| 压力角 | 20° |
| 分度圆直径 | 10.2 |
| 齿顶圆直径 | 11.38 |
| 齿根圆直径 | 8.68 |
| 变位系数 | -0.01 |
| 跨齿数 | 2 |
| 公法线长度 | -0.02 2.79-0.04 |

如果您对说明书有任何疑问或建议，请通过 DocSupport@dji.com 联系我们。

If you have any questions about this document, please contact DJI by sending a message to DocSupport@dji.com.

RoboMaster and ◎sonA5T are trademarks of DJI.

Copyright © 2025 DJI All Rights Reserved.

RoboMaster 和 T 是大疆创新的商标。

Copyright © 2025 大疆创新 版权所有