Support & Community

Tools & Software

Product Folder

Technical

Documents

Order

o  o   Now

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018

SN65HVD23x 3.3-V CAN Bus Transceivers

# 1   Features

•    Operates with a single 3.3 V Supply

•    Compatible With ISO 11898-2 Standard

•    Low Power Replacement for the PCA82C250 Footprint

•    Bus Pin ESD Protection Exceeds ± 16 kV HBM

•    High Input Impedance Allows for Up to 120 Nodes on a Bus

•    Adjustable Driver Transition Times for Improved Emissions Performance

–   SN65HVD230 and SN65HVD231

•    SN65HVD230: Low Current Standby Mode

–   370 μA Typical

•    SN65HVD231: Ultra Low Current Sleep Mode

–   40 nA Typical

•    Designed for Data Rates(1)  up to 1 Mbps

•    Thermal Shutdown Protection

•    Open Circuit Fail-Safe Design

•    Glitch Free Power Up and Power Down Protection for Hot Plugging Applications

(1)      The signaling rate of a line is the number of voltage

transitions that are made per second expressed in the units bps (bits per second).

# 2  Applications

•    Industrial Automation, Control, Sensors and Drive Systems

•    Motor and Robotic Control

•    Building and Climate Control (HVAC)

•    Telecom and Basestation Control and Status

•    CAN Bus Standards Such as CANopen, DeviceNet, and CAN Kingdom

# 3   Description

The SN65HVD230, SN65HVD231, and SN65HVD232 controller   area    network   (CAN)   transceivers   are compatible to the specifications of the  ISO  11898-2 High     Speed     CAN     Physical     Layer     standard (transceiver).  These  devices  are  designed  for  data rates up to 1 megabit per second (Mbps), and include many protection features providing device and CAN network  robustness. The SN65HVD23x transceivers are designed for use with the Texas Instruments 3.3 V µPs, MCUs and DSPs with CAN controllers, or with equivalent  protocol  controller  devices.  The  devices are  intended  for  use  in  applications  employing  the CAN    serial    communication    physical    layer    in accordance with the ISO 11898 standard.

Device Information(1)

| PART NUMBER | PACKAGE | BODY SIZE (NOM) |
| --- | --- | --- |
| SN65HVD230 | SOIC (8) | 4.90 mm × 3.91 mm |
| SN65HVD231 |  |  |
| SN65HVD232 |  |  |

(1)  For all available  packages, see the orderable addendum at the end of the datasheet.

Equivalent Input and Output Schematic Diagrams

VCC                    VREF

B IAS UNIT

D

RS

VCC

VCC

SLOPE CONTROL

VCC

VCC

Thermal Shutdown

VCC / 2

and MODE LOGIC

NC

R

' ·

GND

An IMPORTANT NOTICE at the end of this data sheet addresses availability, warranty, changes, use in safety-critical applications, intellectual property matters and other important disclaimers. PRODUCTION DATA.

# Table of Contents

1    Features 	 1

2    Applications 	 1

3    Description 	 1

4    Revision History	 2

5    Description (continued) 	 4

6    Device Comparison Table 	 4

7    Pin Configuration and Functions 	 5

8    Specifications	 5

8.1   Absolute Maximum Ratings 	 5

8.2   ESD Ratings	 6

8.3   Recommended Operating Conditions 	 6

8.4   Thermal Information 	 6

8.5   Electrical Characteristics: Driver 	 7

8.6   Electrical Characteristics: Receiver 	 7

8.7   Switching Characteristics: Driver 	 8

8.8   Switching Characteristics: Receiver	 8

8.9   Switching Characteristics: Device 	 8

8.10   Device Control-Pin Characteristics 	 9

8.11   Typical Characteristics 	 10

9    Parameter Measurement Information 	 13

10   Detailed Description 	 19

10.1   Overview 	 19

10.2   Functional Block Diagram 	 19

10.3   Feature Description 	 20

10.4   Device Functional Modes	 20

11   Application and Implementation	 25

11.1   Application Information	 25

11.2   Typical Application 	 26

11.3   System Example 	 30

12   Power Supply Recommendations 	 32

13   Layout	 33

13.1   Layout Guidelines 	 33

13.2   Layout Example 	 34

14   Device and Documentation Support 	 35

14.1   Related Links 	 35

14.2   Receiving Notification of Documentation Updates 35

14.3   Community Resources 	 35

14.4   Trademarks 	 35

14.5   Electrostatic Discharge Caution 	 35

14.6   Glossary 	 35

15   Mechanical, Packaging, and Orderable

Information 	 35

| # 4 Revision History NOTE: Page numbers for previous revisions may differ from page numbers in the current version. Changes from Revision N (July 2015) to Revision O Page |  |
| --- | --- |
| • Changed Slope Control Resistance - kW To: Slope Control Resistance - kΩ in Figure 33 22 • Changed Driver Output Signal Slope - V/ms To: Driver Output Signal Slope - V/µs in Figure 33 22 |  |
| Changes from Revision M (May 2015) to Revision N | Page |
| • Changed the data sheet title From; SN65HVD230x 3.3-V CAN Bus Transceivers To: SN65HVD23x 3.3-V CAN Bus Transceivers .......................................................................................................................................................................... 1 |  |
| Changes from Revision L (January 2015) to Revision M | Page |
| • Changed Figure 44 title From: "Layout Example Schematic" To: "SN65HVD23x Board Layout"........................................ 34 |  |
| Changes from Revision K (February 2011) to Revision L | Page |
| • Added Pin Configuration and Functions section, ESD Ratings table, Feature Description section, Device Functional Modes, Application and Implementation section, Power Supply Recommendations section, Layout section, Device and Documentation Support section, and Mechanical, Packaging, and Orderable Information section 1 • Changed the list of Features, Applications, and Description 1 • Added THERMAL SHUTDOWN TEMPERATURE AND HYSTERESIS in the Recommended Operating Conditions table. 6 • Added the THERMAL SHUTDOWN paragraph to the Application Information section 20 • Added Figure 34 and Figure 35 25 • Added the CAN TERMINATION paragraph to the Application Information section 26 • Added the BUS LOADING, LENGTH AND NUMBER OF NODES paragraph to the Application Information section 28 |  |

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

| Changes from Revision J (January 2009) to Revision K | Page |
| --- | --- |
| • Replaced the DISSIPATION RATING TABLE with the Thermal Information table................................................................ 6 |  |
| Changes from Revision I (October 2007) to Revision J | Page |

•     Deleted Low-to-High Propagation Delay Time vs Common-Mode Input Voltage Characteristics 	 12

•     Deleted Driver Schematic Diagram 	 12

•    Added Figure 38 	 32

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

# 5   Description (continued)

Designed for operation in especially harsh environments, these devices feature cross wire protection, loss of ground  and  overvoltage  protection,  overtemperature  protection,  as  well  as  wide  common  mode  range  of operation.

The CAN transceiver is the CAN physical layer and interfaces the single ended host CAN protocol controller with the differential CAN bus found in industrial, building automation, and automotive applications. These devices operate over a -2 V to 7 V common mode range on the bus, and can withstand common mode transients of ±25 V.

The RS  pin (pin 8) on the SN65HVD230 and SN65HVD231 provides three different modes of operation: high speed  mode,  slope  control  mode,  and  low-power  mode.  The  high  speed  mode  of  operation  is  selected  by connecting the  RS   pin to ground,  allowing the transmitter output transistors to switch on and off as fast as possible with no limitation on the rise and fall slopes. The rise and fall slopes can also be adjusted by connecting a resistor in series between the RS  pin and ground. The slope will be proportional to the pin's output current. With a resistor value of 10 kΩ the device will have a slew rate of ~15 V/μs, and with a resistor value of 100 kΩ the device will have a slew rate of ~2 V/μs. See Application Information for more information.

The SN65HVD230 enters a low current standby mode (listen only) during which the driver is switched off and the receiver  remains  active  if  a  high  logic  level  is  applied  to  the  RS   pin.  This  mode  provides  a  lower  power consumption  mode  than  normal  mode  while  still  allowing  the  CAN  controller  to  monitor  the  bus  for  activity indicating it should return the transceiver to normal mode or slope control mode. The host controller (MCU, DSP) returns the device to a transmitting mode (high speed or slope control) when it wants to transmit a message to the bus or if during standby mode it received bus traffic indicating the need to once again be ready to transmit.

The difference between the SN65HVD230 and the SN65HVD231 is that both the driver and the receiver are switched off in the SN65HVD231 when a high logic level is applied to the RS  pin. In this sleep mode the device will not be able to transmit messages to the bus or receive messages from the bus. The device will remain in sleep mode until it is reactivated by applying a low logic level on the RS  pin.

# 6   Device Comparison Table

| PART NUMBER(1) | LOW POWER MODE | INTEGRATED SLOPE CONTROL | Vref PIN | TA | MARKED AS: |
| --- | --- | --- | --- | --- | --- |
| SN65HVD230 | Standby mode | Yes | Yes | 40°C to 85°C | VP230 |
| SN65HVD231 | Sleep mode | Yes | Yes |  | VP231 |
| SN65HVD232 | No standby or sleep mode | No | No |  | VP232 |

(1)    For the most current package and ordering information, see Mechanical, Packaging, and Orderable Information, or see the TI web site at www.ti.com.

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

# 7   Pin Configuration and Functions

1                               8

2                               7

3                               6

4                               5

SN65HVD230D (Marked as VP230)

SN65HVD231D (Marked as VP231)

Top View

RS

CANH

CANL

Vref

D

GND

V CC

R

Not to scale

D

GND

V CC

R

Not to scale

1                               8                NC

2                               7                 CANH

3                               6                 CANL

4                               5                  NC

SN65HVD232D (Marked as VP232)

Top View

Pin Functions

| PIN |  | TYPE | DESCRIPTION |
| --- | --- | --- | --- |
| NAME | NO. |  |  |
| D | 1 | I | CAN transmit data input (LOW for dominant and HIGH for recessive bus states), also called TXD, driver input |
| GND | 2 | GND | Ground connection |
| V CC | 3 | Supply | Transceiver 3.3V supply voltage |
| R | 4 | O | CAN receive data output (LOW for dominant and HIGH for recessive bus states), also called RXD, receiver output |
| V ref | 5 | O | SN65HVD230 and SN65HVD231: V CC / 2 reference output pin |
| NC |  | NC | SN65HVD232: No Connect |
| CANL | 6 | I/O | Low level CAN bus line |
| CANH | 7 | I/O | High level CAN bus line |
| RS | 8 | I | SN65HVD230 and SN65HVD231: Mode select pin: strong pull down to GND = high speed mode, strong pull up to V CC = low power mode, 10kΩ to 100kΩ pull down to GND = slope control mode |
| NC |  | I | SN65HVD232: No Connect |

# 8  Specifications

## 8.1  Absolute Maximum Ratings

over operating free-air temperature range (unless otherwise noted)(1)(2)

|  | MIN MAX | UNIT |
| --- | --- | --- |
| Supply voltage, V CC | –0.3 6 | V |
| Voltage at any bus terminal (CANH or CANL) | –4 16 | V |
| Voltage input, transient pulse, CANH and CANL, through 100 Ω (see Figure 24) | –25 25 | V |
| Digital Input and Output voltage, VI (D or R) | –0.5 V CC + 0.5 | V |
| Receiver output current, IO | –11 11 | mA |
| Continuous total power dissipation | See Thermal Information |  |
| Storage temperature, Tstg | –40 85 | °C |

(1)   Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated under Recommended Operating Conditions is not implied. Exposure to absolute-maximum-rated conditions for extended periods may affect device reliability.

(2)   All voltage values, except differential I/O bus voltages, are with respect to network ground terminal.

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

## 8.2   ESD Ratings

|  |  |  |  | VALUE | UNIT |
| --- | --- | --- | --- | --- | --- |
| Electrostatic V(ESD) discharge | Human 001 (1) | body model (HBM), per ANSI/ESDA/JEDEC JS- | CANH, CANL and GND | ± 16000 | V |
|  |  |  | All pins | ±4000 |  |
|  | Charged-device model (CDM), per JEDEC specification JESD22-C101 (2) |  |  | ± 1000 |  |

(1)   JEDEC document JEP155 states that 500-V HBM allows safe manufacturing with a standard ESD control process.

(2)   JEDEC document JEP157 states that 250-V CDM allows safe manufacturing with a standard ESD control process.

## 8.3   Recommended Operating Conditions

|  |  | MIN NOM MAX | UNIT |
| --- | --- | --- | --- |
| Supply voltage, V CC |  | 3 3.6 | V |
| Voltage at any bus terminal (common mode) VIC |  | –2(1) 7 | V |
| Voltage at any bus terminal (separately) VI |  | –2.5 7.5 | V |
| High-level input voltage, VIH | D, R | 2 | V |
| Low-level input voltage, VIL | D, R | 0.8 | V |
| Differential input voltage, VID (see Figure 22) |  | –6 6 | V |
| Input voltage, V(Rs) |  | 0 V CC | V |
| Input voltage for standby or sleep, V(Rs) |  | 0.75 VCC V CC | V |
| Wave-shaping resistance, Rs |  | 0 100 | kΩ |
| High-level output current, I OH | Driver | –40 | mA |
|  | Receiver | –8 |  |
| Low-level output current, I OL | Driver | 48 | mA |
|  | Receiver | 8 |  |
| Thermal shutdown temperature |  | 165 | °C |
| Thermal shutdown hysteresis |  | 10 |  |
| Operating free-air temperature, TA |  | –40 85 |  |

(1)   The algebraic convention, in which the least positive (most negative) limit is designated as minimum is used in this data sheet.

## 8.4   Thermal Information

| THERMAL METRIC(1) | SN65HVD230 | SN65HVD231 | SN65HVD232 | UNIT |
| --- | --- | --- | --- | --- |
|  | D |  |  |  |
|  | 8 PINS |  |  |  |
| RθJA Junction-to-ambient thermal resistance | 76.8 | 101.5 | 101.5 | °C/W |
| RθJC(top) Junction-to-case (top) thermal resistance | 33.4 | 43.3 | 43.3 | °C/W |
| RθJB Junction-to-board thermal resistance | 15.3 | 42.2 | 42.4 | °C/W |
| ψJT Junction-to-top characterization parameter | 1.4 | 4.8 | 4.8 | °C/W |
| ψJB Junction-to-board characterization parameter | 14.9 | 41.8 | 41.8 | °C/W |

(1)    For more information about traditional and new thermal metrics, see the Semiconductor and IC Package Thermal Metrics application report.

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

## 8.5   Electrical Characteristics: Driver

over recommended operating conditions (unless otherwise noted)

| PARAMETER |  |  |  | TEST CONDITIONS |  | MIN TYP(1) MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V OH | Bus output voltage | Dominant |  | VI = 0 V, See Figure 18 and Figure 20 | CANH | 2.45 V CC | V |
|  |  |  |  |  | CANL | 0.5 1.25 |  |
| VOL |  | Recessive |  | VI = 3 V, See Figure 18 and Figure 20 | CANH | 2.3 |  |
|  |  |  |  |  | CANL | 2.3 |  |
| VOD(D) | Differential output voltage | Dominant |  | VI = 0 V, See Figure 18 |  | 1.5 2 3 | V |
|  |  |  |  | VI = 0 V, See Figure 19 |  | 1.2 2 3 |  |
| VOD(R) |  | Recessive |  | VI = 3 V, See Figure 18 |  | –120 0 12 | mV |
|  |  |  |  | VI = 3 V, No load |  | –0.5 –0.2 0.05 | V |
| IIH High-level input current |  |  |  | VI = 2 V |  | –30 | μA |
| IIL Low-level input current |  |  |  | VI = 0.8 V |  | –30 | μA |
| IOS Short-circuit output current |  |  |  | VCANH = -2 V |  | –250 250 | mA |
|  |  |  |  | VCANL = 7 V |  | –250 250 |  |
| Co Output capacitance |  |  |  | See receiver |  |  |  |
| ICC | Supply current | Standby | SN65HVD230 | V(Rs) = V CC |  | 370 600 | μA |
|  |  | Sleep | SN65HVD231 | V(Rs) = V CC, D at V CC |  | 0.04 1 |  |
|  |  | All devices | Dominant | VI = 0 V, No load | Dominant | 10 17 | mA |
|  |  |  | Recessive | VI = V CC, No load | Recessive | 10 17 |  |

(1)   All typical values are at 25°C and with a 3.3-V supply.

## 8.6   Electrical Characteristics: Receiver

over recommended operating conditions (unless otherwise noted)

| PARAMETER | TEST CONDITIONS |  | MIN TYP(1) MAX | UNIT |
| --- | --- | --- | --- | --- |
| VIT+ Positive-going input threshold voltage | See Table 1 |  | 750 900 | mV |
| VIT- Negative-going input threshold voltage |  |  | 500 650 | mV |
| Vhys Hysteresis voltage (VIT+ – VIT–) |  |  | 100 |  |
| V OH High-level output voltage | –6 V ≤ VID ≤ 500 mV, IO = –8 mA, See Figure 22 |  | 2.4 | V |
| VOL Low-level output voltage | 900 mV ≤ VID ≤ 6 V, IO = 8 mA, See Figure 22 |  | 0.4 |  |
| II Bus input current | VIH = 7 V | Other input at 0 V, D = 3 V | 100 250 | μA |
|  | VIH = 7 V, V CC = 0 V |  | 100 350 |  |
|  | VIH = -2 V |  | –200 –30 | μA |
|  | VIH = -2 V, V CC = 0 V |  | –100 –20 |  |
| CI CANH, CANL input capacitance | Pin-to-ground, V(D) = 3 V, VI = 0.4 sin(4E6πt) + 0.5 V |  | 32 | pF |
| C Diff Differential input capacitance | Pin-to-pin, VI = 0.4 sin(4E6πt) + 0.5 V | V(D) = 3 V, | 16 | pF |
| R Diff Differential input resistance | Pin-to-pin, V(D) = 3 V |  | 40 70 100 | kΩ |
| RI CANH, CANL input resistance |  |  | 20 35 50 | kΩ |
| ICC Supply current | See driver |  |  |  |

(1)   All typical values are at 25°C and with a 3.3-V supply.

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

## 8.7   Switching Characteristics: Driver

over recommended operating conditions (unless otherwise noted)

| PARAMETER |  |  | TEST CONDITIONS | MIN TYP MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| SN65HVD230 AND SN65HVD231 |  |  |  |  |  |
| tPLH | Propagation delay time, low-to-high-level output | V(Rs) = 0 V | CL = 50 pF, See Figure 21 | 35 85 | ns |
|  |  | RS with 10 kΩ to ground |  | 70 125 |  |
|  |  | RS with 100 kΩ to ground |  | 500 870 |  |
| tPHL | Propagation delay time, high-to-low-level output | V(Rs) = 0 V |  | 70 120 | ns |
|  |  | RS with 10 kΩ to ground |  | 130 180 |  |
|  |  | RS with 100 kΩ to ground |  | 870 1200 |  |
| tsk(p) Pulse skew (¦tPHL - tPLH¦) |  | V(Rs) = 0 V |  | 35 | ns |
|  |  | RS with 10 kΩ to ground |  | 60 |  |
|  |  | RS with 100 kΩ to ground |  | 370 |  |
| tr Differential output signal rise time |  | V(Rs) = 0 V |  | 25 50 100 | ns |
| tf Differential output signal fall time |  |  |  | 40 55 80 | ns |
| tr Differential output signal rise time |  | RS with 10 kΩ to ground |  | 80 120 160 | ns |
| tf Differential output signal fall time |  |  |  | 80 125 150 | ns |
| tr Differential output signal rise time |  | RS with 100 kΩ to ground |  | 600 800 1200 | ns |
| tf Differential output signal fall time |  |  |  | 600 825 1000 | ns |
| SN65HVD232 |  |  |  |  |  |
| tPLH Propagation delay time, low-to-high-level output |  |  | CL = 50 pF, See Figure 21 | 35 85 | ns |
| tPHL Propagation delay time, high-to-low-level output |  |  |  | 70 120 |  |
| tsk(p) Pulse skew (¦tPHL - tPLH¦) |  |  |  | 35 |  |
| tr Differential output signal rise time |  |  |  | 25 50 100 |  |
| tf Differential output signal fall time |  |  |  | 40 55 80 |  |

## 8.8   Switching Characteristics: Receiver

over recommended operating conditions (unless otherwise noted)

| PARAMETER | TEST CONDITIONS | MIN TYP MAX | UNIT |
| --- | --- | --- | --- |
| tPLH Propagation delay time, low-to-high-level output | See Figure 23 | 35 50 | ns |
| tPHL Propagation delay time, high-to-low-level output |  | 35 50 | ns |
| tsk(p) Pulse skew (¦tPHL - tPLH¦) |  | 10 | ns |
| tr Output signal rise time | See Figure 23 | 1.5 | ns |
| tf Output signal fall time |  | 1.5 | ns |

## 8.9   Switching Characteristics: Device

over recommended operating conditions (unless otherwise noted)

| PARAMETER |  | TEST CONDITIONS | MIN TYP MAX | UNIT |
| --- | --- | --- | --- | --- |
| t(LOOP1) Toouttapul lto, orpecesslaiyve, dritoeorminipunatto receiver |  | V(Rs) = 0 V, See Figure 26 | 70 115 | ns |
|  |  | RS with 10 kΩ to ground, See Figure 26 | 105 175 |  |
|  |  | RS with 100 kΩ to ground, See Figure 26 | 535 920 |  |
| t(LOOP2) | Total loop delay, driver input to receiver output, dominant to recessive | V(Rs) = 0 V, See Figure 26 | 100 135 | ns |
|  |  | RS with 10 kΩ to ground, See Figure 26 | 155 185 |  |
|  |  | RS with 100 kΩ to ground, See Figure 26 | 830 990 |  |

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

## 8.10   Device Control-Pin Characteristics

over recommended operating conditions (unless otherwise noted)

| PARAMETER |  | TEST CONDITIONS | MIN TYP(1) MAX | UNIT |
| --- | --- | --- | --- | --- |
| t(WAKE) | SN65HVD230 wake-up time from standby mode with RS | See Figure 25 | 0.55 1.5 | μs |
|  | SN65HVD231 wake-up time from sleep mode with RS |  | 3 5 | μs |
| V ref Reference output voltage |  | -5 μA < I(Vref) < 5 μA | 0.45 V CC 0.55 V CC | V |
|  |  | -50 μA < I(Vref) < 50 μA | 0.4 V CC 0.6 V CC |  |
| I(Rs) Input current for high-speed |  | V(Rs) < 1 V | –450 0 | μA |

(1)   All typical values are at 25°C and with a 3.3-V supply.

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

C

C

8.11   Typical Characteristics

| ICC − Supply Current (RMS) − mA 22 21 20 19 18 17 16 15 14 13 | V CC = 3.3 | V |  |  |  | I I(L) − Logic Input Current − μ A 0 −2 −4 −6 −8 −10 −12 −14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60-Ω Loa R S at 0 V | d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 250 500 750 1000 f − Frequency − kbps Figure 1. Supply Current (RMS) vs Frequency |  |  |  |  |  | −16 0 0.6 1.1 1.6 2.1 2.6 3.1 3.6 VI − Input Voltage − V Figure 2. Logic Input Current (Pin D) vs Input Voltage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I I _ Bus Input Current _ μ A 400 300 200 100 0 _100 _200 _300 _400 V CC; = 0 V. V CC; = 3.; 6 V. _7 _6 _4 _3 _1 0 1 3 4 6 7 8 10 11 12 VI _ Bus Input Voltage _ V Figure 3. Bus Input Current vs Bus Input Voltage | I OL− Driver Low-Level Output Current − mA 160 140 120 100 80 60 40 20 0 0 1 2 3 4 VO(CANL)− Low-Level Output Voltage − V Figure 4. Driver Low-Level Output Current vs Low-Level Output Voltage |
| I OH − Driver High-Level Output Current − mA 120 100 80 60 40 20 0 0 0.5 1 1.5 2 2.5 3 3.5 VO(CANH) − High-Level Output Voltage − V Figure 5. Driver High-Level Output Current vs High-Level Output Voltage |  |  |  |  |  | VOD− Dominant Voltage − V 3 2.5 2 1.5 1 0.5 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | VC | C = 3.6 |  | V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | V V | C = 3.3 V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | C = 3 V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C Figure 6. Dominant Voltage (VOD) vs Free-Air Temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

Typical Characteristics (continued)

| tPLH − Receiver Low-to-High Propagation Delay Time − ns 38 RS = 0 37 36 V CC = 3 V 35 V CC = 3.3 V 34 V CC = 3.6 V 33 32 31 30 −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C | tPHL− Receiver High-to-Low Propagation Delay Time − ns 40 RS = 0 39 38 V CC = 3 V 37 V CC = 3.3 V 36 V CC = 3.6 V 35 34 −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C |
| --- | --- |
| Figure 7. Receiver Low-to-High Propagation Delay Time vs Free-Air Temperature | Figure 8. Receiver High-to-Low Propagation Delay Time vs Free-Air Temperature |
| tPLH − Driver Low-to-High Propagation Delay Time − ns 55 RS = 0 V CC = 3 V 50 45 V CC = 3.3 V 40 35 V CC = 3.6 V 30 25 20 15 10 −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C | tPHL− Driver High-to-Low Propagation Delay Time − ns 90 RS = 0 85 V CC = 3.6 V 80 75 V CC = 3.3 V 70 V CC = 3 V 65 60 55 50 −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C |
| Figure 9. Driver Low-to-High Propagation Delay Time vs Free-Air Temperature | Figure 10. Driver High-to-Low Propagation Delay Time vs Free-Air Temperature |
| tPLH − Driver Low-to-High Propagation Delay Time − ns 90 RS = 10 kΩ 80 V CC = 3 V 70 V CC = 3.3 V 60 V CC = 3.6 V 50 40 30 20 10 0 −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C | tPHL − Driver High-to-Low Propagation Delay Time − ns 150 RS = 10 kΩ V CC = 3.6 V 140 V CC = 3.3 V 130 V CC = 3 V 120 110 100 90 80 −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C |
| Figure 11. Driver Low-to-High Propagation Delay Time vs Free-Air Temperature | Figure 12. Driver High-to-Low Propagation Delay Time vs Free-Air Temperature |

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

Typical Characteristics (continued)

| tPLH − Driver Low-to-High Propagation Delay Time − ns | 800 700 600 500 400 300 200 100 0 | RS =; 100 kΩ. V; CC = 3 V. V CC = 3; .3 V. V; = 3.6. V. CC. −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C | tPHL− Driver High-to-Low Propagation Delay Time − ns |  |  |  | 1000 950 900 850 800 750 700 RS =; 100 kΩ. V CC = 3; V CC .3 V; = 3.6 V. V CC = 3; V. −55 −40 0 25 70 85 125 TA − Free-Air Temperature − °C |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Figure 13. Driver Low-to-High Propagation Delay Time vs Free-Air Temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Figure 14. Driver High-to-Low Propagation Delay Time vs Free-Air Temperature |  |  |  |  |  |  |  |  |
| IO − Driver Output Current − mA | 50 40 30 20 10 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | tf _ Differential Driver Output Fall Time _ μ s |  |  |  | 1.50 1.40 1.30 1.20 1.10 1.00 0.90 0.80 0.70 0.60 0.50 0.40 0.30 0.20 0.10 0 |  | V CC =; 3.3 V. V CC =; 3.6 V. V; CC = 3 V. 0 50 100 150 200 Rs _ Source Resistance _ kΩ |
| 1 1.5 2 2.5 3 3.5 4 V CC − Supply Voltage − V Figure 15. Driver Output Current vs Supply Voltage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Figure 16. Differential Driver Output Fall Time vs Source Resistance (Rs) |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Current |
| V ref _ Reference Voltage _ V |  |  |  |  |  |  |  |  |  |  |  |  | 2.5 2 1.5 1 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | V CC | = 3.6 V |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | V CC |  | =3V |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| _50 _5 5 50 I ref _ Reference Current _ μA Figure 17. Reference Voltage vs Reference |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

# 9   Parameter Measurement Information

V CC

IO

II

D

b

VOD

IO

VI

0 V or 3 V                                                      60 Ω

CANH

CANL

Figure 18.  Driver Voltage and Current Definitions

0 V

167 Ω

60 Ω

167 Ω

±   –2 V ≤ V TEST ≤ 7 V

VOD

Figure 19.  Driver VOD

CANL                                                  ≈ 1 V                   VOH        CANL

Recessive

≈ 2.3 V

Dominant CANH

≈ 3 V                     V OH

CANH

VOL

Figure 20.  Driver Output Voltage Definitions

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

Parameter Measurement Information (continued)

RL = 60 Ω

50 Ω

RS = 0 Ω to 100 kΩ for SN65HVD230 and SN65HVD231 N/A for SN65HVD232

Signal

Generator (see Note A)

CL = 50 pF (see Note B)

VO

3 V

1.5 V

0 V tPHL

90%

0.9 V

0.5 V

10%

Input

Output

VOD(D)

VOD(R)

tr                                                                             t f

tPLH

A.    The input pulse is supplied by a generator having the following characteristics: PRR ≤ 500 kHz, 50% duty cycle, tr ≤ 6 ns, tf ≤ 6 ns, Zo = 50 Ω .

B.    CL includes probe and jig capacitance.

Figure 21.  Driver Test Circuit and Voltage Waveforms

IO VID

VCANH

VCANL

VCANH + VCANL

2

VIC =

VO

Figure 22.  Receiver Voltage and Current Definitions

SN65HVD230, SN65HVD231, SN65HVD232 www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

Parameter Measurement Information (continued)

Output

1.5 V

Signal

Generator (see Note A)

CL = 15 pF (see Note B)

50 Ω

90%

Output                                                                                                   1.3 V

10% tr                                                                             t f

Input

tPLH                                                                                   t PHL

2.9 V

2.2 V

1.5 V

V OH

VOL

A.    The input pulse is supplied by a generator having the following characteristics: PRR ≤ 500 kHz, 50% duty cycle, tr ≤ 6 ns, tf ≤ 6 ns, Zo = 50 Ω .

B.    CL includes probe and jig capacitance.

Figure 23.  Receiver Test Circuit and Voltage Waveforms

Pulse Generator,

15 μs Duration,

1% Duty Cycle

100 Ω

Figure 24.  Overvoltage Protection

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

Parameter Measurement Information (continued)

Table 1. Receiver Characteristics Over Common Mode With V(Rs) = 1.2 V

| VIC | VID | VCANH | VCANL | R OUTPUT |  |
| --- | --- | --- | --- | --- | --- |
| -2 V | 900 mV | -1.55 V | -2.45 V | L | VOL |
| 7 V | 900 mV | 8.45 V | 6.55 V | L |  |
| 1 V | 6 V | 4 V | -2 V | L |  |
| 4 V | 6 V | 7 V | 1 V | L |  |
| -2 V | 500 mV | -1.75 V | -2.25 V | H | V OH |
| 7 V | 500 mV | 7.25 V | 6.75 V | H |  |
| 1 V | -6 V | -2 V | 4 V | H |  |
| 4 V | -6 V | 1 V | 7 V | H |  |
| X | X | Open | Open | H |  |

CL = 15 pF

V CC

60 Ω <

O

0 V

Output

10 kΩ

D

R

RS

+

50 Ω         V(Rs) –

Generator

PRR = 150 kHz

50% Duty Cycle

tr, t f < 6 ns

Zo = 50 Ω

Signal

Generator

R Output

V CC

V(Rs)                                                                                                                                 1.5 V

0 V

t(WAKE)

1.3 V

Figure 25.  t(WAKE) Test Circuit and Voltage Waveforms

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

100 kΩ ±5%

RS

CANH

D

60 Ω ±1%

VI

CANL

R

+

15 pF ±20%

VO

0 Ω, 10 kΩ or

DUT

VI            50%              50%

t(LOOP1)

t(LOOP2)

50% +

VO

50%f

V CC

0 V

V OH

VOL

A.    All VI  input  pulses are supplied  by a generator having the following characteristics: tr  or tf  ≤ 6 ns, Pulse Repetition Rate (PRR) = 125 kHz, 50% duty cycle.

Figure 26.  t(LOOP) Test Circuit and Voltage Waveforms

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

CANH and CANL Inputs

Input

16 V                        110 kΩ                    9 kΩ

45 kΩ

20 V                            9 kΩ

V CC

D Input

V CC

100 kΩ

1 kΩ Input

9 V

CANH and CANL Outputs

V CC

V CC

16 V

Output

20 V

R Output

5 Ω

Output

9 V

Figure 27.  Equivalent Input and Output Schematic Diagrams

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

# 10   Detailed Description

## 10.1   Overview

Implementation

| TMS320Lx2403/6/7 3.3-V DSP |  |  |
| --- | --- | --- |
| Embedded CAN Controller |  |  |
| SN65HVD230 |  |  |
| CAN Bus-Line |  |  |

ISO  11898 family  of standards are the  international standard for high speed serial communication  using the controller area network (CAN) bus protocol and physical layers (transceivers). It supports multimaster operation, real time control, programmable data rates up to 1 Mbps, and powerful redundant error checking procedures that provide reliable data transmission. It is suited for networking intelligent devices as well as sensors and actuators within the rugged electrical environment of a machine chassis or factory floor. The SN65HVD23x family of 3.3 V CAN transceivers implement the lowest layers of the ISO/OSI reference model, the ISO11898-2 standard. This is the interface with the physical signaling output of the CAN controller of the Texas Instruments µPs, MCUs and DSPs, such as TMS320Lx240x 3.3 V DSPs, as illustrated in Figure 28.

ISO 11898 Specification

| Application Specific Layer |  |
| --- | --- |
| Data-Link Layer | Logic Link Control |
|  | Medium Access Control |
| Physical Layer | Physical Signaling |
|  | Physical Medium Attachment |
|  | Medium Dependent Interface |

Figure 28.  Layered ISO 11898 Standard Architecture

SN65HVD232

Logic Diagram (Positive Logic)

10.2   Functional Block Diagram

1

D

7

CANH CANL

4

R

6

SN65HVD230, SN65HVD231 Logic Diagram (Positive Logic)

Vref

CANH

CANL

V CC

D

RS

R

5

7

6

3

1

8

4

Figure 29.  Logic Diagram (Positive Logic)

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018

www.ti.com

## 10.3   Feature Description

The SN65HVD230/231/232 are pin-compatible (but not functionally identical) with one another and, depending upon the application, may be used with identical circuit boards.

These transceivers feature single 3.3 V supply operation and standard compatibility with signaling rates up to 1 Mbps, ± 16 kV HBM ESD protection on the bus pins, thermal shutdown protection, bus fault protection, and open- circuit receiver failsafe. The fail-safe design of the receiver assures a logic high at the receiver output if the bus wires become open circuited.

The bus pins are also maintained in a high-impedance state during low VCC  conditions to ensure glitch-free power-up  and  power-down  bus  protection  for  hot-plugging  applications.  This  high-impedance  condition  also means that an unpowered node does not disturb the bus. Transceivers without this feature usually have a very low output impedance. This results in a high current demand when the transceiver is unpowered, a condition that could affect the entire bus.

### 10.3.1   Vref Voltage Reference

The Vref  pin (pin 5) on the SN65HVD230 and SN65HVD231 is available as a VCC/2 voltage reference. This pin can be connected to the common mode point of a split termination to help further stabilize the common mode voltage of the bus. If the Vref  pin is not used it may be left floating.

### 10.3.2   Thermal Shutdown

If a high ambient temperature or excessive output currents result in thermal shutdown, the driver will be disabled and the  bus  pins  become  high  impedance.  During  thermal  shutdown  the  D  pin  to  bus  transmission  path  is blocked and the CAN bus pins are high impedance and biased to a recessive level. Once the thermal shutdown condition is cleared and the junction temperature drops below the thermal shutdown temperature the driver will be reactivated and resume normal operation. During a thermal shutdown the receiver to R pin path remains operational.

## 10.4   Device Functional Modes

The RS  pin (Pin 8) of the SN65HVD230 and SN65HVD231 provides three different modes of operation: high- speed mode, slope-control mode, and low-power mode.

### 10.4.1   High-Speed Mode

The high-speed mode can be selected by applying a logic low to the RS  pin  (pin 8). The high-speed mode of operation is commonly employed in industrial applications. High-speed allows the output to switch as fast as possible with no internal limitation on the output rise and fall slopes. If the high speed transitions are a concern for emissions performance slope control mode can be used.

If both high speed mode and the low-power standby mode is to be used in the application, direct connection to a µP, MCU or DSP general purpose output pin can be used to switch between a logic-low level (< 1.2 V) for high speed operation, and the logic-high level (> 0.75 VCC) for standby. Figure 30 shows a typical DSP connection, and Figure 31 shows the HVD230 driver output signal in high-speed mode on the CAN bus.

IOPF6

TMS320LF2406

or

TMS320LF2407

RS

CANH

CANL

Vref

D

GND

V CC

R

8

7

6

5

1

2

3

4

Figure 30.  RS (Pin 8) Connection to a TMS320LF2406/07 for High Speed/Standby Operation

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018 Device Functional Modes (continued)

1 Mbps Driver Output NRZ Data

1

Figure 31.  Typical High Speed SN65HVD230 Output Waveform into a 60-Ω Load

### 10.4.2   Slope Control Mode

Electromagnetic compatibility is essential in many applications while still making use of unshielded twisted pair bus  cable  to  reduce  system  cost.  Slope  control  mode  was  added  to  the  SN65HVD230  and  SN65HVD231 devices to reduce the electromagnetic interference produced by the rise and fall times of the driver and resulting harmonics. These rise and fall slopes of the driver outputs can be adjusted by connecting a resistor from RS  (pin 8) to ground or to a logic low voltage, as shown in Figure 32. The slope of the driver output signal is proportional to the pin's output current. This slope control is implemented with an external resistor value of 10 kΩ to achieve a ~15 V/μs slew rate, and up to 100 kΩ to achieve a ~2.0 V/μs slew rate as displayed in Figure 33.

IOPF6

TMS320LF2406

or

TMS320LF2407

RS                100 kΩ

CANH

CANL

Vref

D

GND

V CC

R

8

7

6

5

1

2

3

4

10 kΩ

to

Figure 32.  Slope Control/Standby Connection to a DSP

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018

www.ti.com

Device Functional Modes (continued)

Driver Outout Signa l Slope – V/μs

0

0      .07      0.8      0           2520                      00

Slope Control Resistance – kΩ

25

20

15

10

5

Figure 33.  HVD230 Driver Output Signal Slope vs Slope Control Resistance Value

### 10.4.3   Standby Mode (Listen Only Mode) of the HVD230

If a logic high (> 0.75 VCC) is applied to RS  (pin 8) in Figure 30 and Figure 32, the circuit of the SN65HVD230 enters a low-current, listen only standby mode, during which the driver is switched off and the receiver remains active. In this listen only state, the transceiver is completely passive to the bus. It makes no difference if a slope control resistor is in place as shown in Figure 32. The µP can reverse this low-power standby mode when the rising edge of a dominant state (bus differential voltage > 900 mV typical) occurs on the bus. The µP, sensing bus activity, reactivates the driver circuit by placing a logic low (< 1.2 V) on RS  (pin 8).

### 10.4.4   The Babbling Idiot Protection of the HVD230

Occasionally, a runaway CAN controller unintentionally sends messages that completely tie up the bus (what is referred to in CAN jargon as a babbling idiot). When this occurs, the µP, MCU or DSP can engage the listen-only standby  mode  of the transceiver to disable the driver and  release the  bus,  even when  access to the  CAN controller has been lost. When the driver circuit is deactivated, its outputs default to a high-impedance state (recessive).

### 10.4.5   Sleep Mode of the HVD231

The unique difference between the SN65HVD230 and the SN65HVD231 is that both driver and receiver are switched off in the SN65HVD231 when a logic high is applied to RS  (pin 8). The device remains in a very low power-sleep mode until the circuit is reactivated with a logic low applied to RS  (pin 8). While in this sleep mode, the bus-pins are in a high-impedance state, while the D and R pins default to a logic high.

### 10.4.6   Summary of Device Operating Modes

Table 2 shows a summary of the operating modes for the SN65HVD230 and SN65HVD231. Please note that the SN65HVD232 is a basic CAN transceiver has only the normal high speed mode of operation; pins 5 and 8 are no connection (NC).

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

Device Functional Modes (continued)

Table 2. SN65HVD230 and SN65HVD231 Operating Modes

| R S Pin | MODE |  | DRIVER | RECEIVER | RXD Pin |
| --- | --- | --- | --- | --- | --- |
| LOW, V(Rs) < 1.2 V, strong pull down to GND | High Speed Mode |  | Enabled (ON) High Speed | Enabled (ON) | Mirrors Bus State(1) |
| LOW, V(Rs) < 1.2 V, 10 kΩ to 100 kΩ pull down to GND | Slope Control Mode |  | Enabled (ON) with Slope Control | Enabled (ON) | Mirrors Bus State |
| HIGH, V(Rs) > 0.75 V CC | Low Current Mode | SN65HVD230: Standby Mode | Disabled (OFF) | Enabled (ON) | Mirrors Bus State |
|  |  | SN65HVD231: Sleep Mode |  | Disabled (OFF) | High |

(1)    Mirrors bus state: low if CAN bus is dominant, high if CAN bus is recessive.

Table 3. SN65HVD230 and SN65HVD231 Driver Functions

| DRIVER (SN65HVD230, SN65HVD231)(1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| INPUT D | RS | OUTPUTS |  | BUS STATE |
|  |  | CANH | CANL |  |
| L | V(Rs) < 1.2 V (including 10 kΩ to 100 kΩ pull down to GND) | H | L | Dominant |
| H |  | Z | Z | Recessive |
| Open | X | Z | Z | Recessive |
| X | V(Rs) > 0.75 V CC | Z | Z | Recessive |

(1)    H = high level; L = low level; X = irrelevant; ? = indeterminate; Z = high impedance

Table 4. SN65HVD230 Receiver Functions

| RECEIVER (SN65HVD230)(1) |  |  |
| --- | --- | --- |
| DIFFERENTIAL INPUTS | RS | OUTPUT R |
| VID ≥ 0.9 V | X | L |
| 0.5 V < VID < 0.9 V | X | ? |
| VID ≤ 0.5 V | X | H |
| Open | X | H |

(1)    H = high level; L = low level; X = irrelevant; ? = indeterminate

Table 5. SN65HVD231 Receiver Functions

| RECEIVER (SN65HVD231)(1) |  |  |
| --- | --- | --- |
| DIFFERENTIAL INPUTS | RS | OUTPUT R |
| VID ≥ 0.9 V | V(Rs) < 1.2 V (including 10 kΩ to 100 kΩ pull down to GND) | L |
| 0.5 V < VID < 0.9 V |  | ? |
| VID ≤ 0.5 V |  | H |
| X | V(Rs) > 0.75 V CC | H |
| X | 1.2 V < V(Rs) < 0.75 V CC | ? |
| Open | X | H |

(1)    H = high level; L = low level; X = irrelevant; ? = indeterminate

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018                                                                                                                                                       www.ti.com

Table 6. SN65HVD232 Receiver Functions

| RECEIVER (SN65HVD232)(1) |  |
| --- | --- |
| DIFFERENTIAL INPUTS | OUTPUT R |
| VID ≥ 0.9 V | L |
| 0.5 V < VID < 0.9 V | ? |
| VID ≤ 0.5 V | H |
| Open | H |

(1)    H = high level; L = low level; X = irrelevant; ? = indeterminate

Table 7. SN65HVD232 Driver Functions

| DRIVER (SN65HVD232)(1) |  |  |  |
| --- | --- | --- | --- |
| INPUT D | OUTPUTS |  | BUS STATE |
|  | CANH | CANL |  |
| L | H | L | Dominant |
| H | Z | Z | Recessive |
| Open | Z | Z | Recessive |

(1)    H = high level; L = low level; Z = high impedance

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

# 11  Application and Implementation

NOTE

Information  in  the  following  applications  sections  is  not  part  of  the   TI  component specification, and TI does not warrant its accuracy or completeness. TI’s customers are responsible for determining suitability of components for their purposes. Customers should validate and test their design implementation to confirm system functionality.

## 11.1  Application Information

This application section provides information concerning the implementation of the physical medium attachment layer in a CAN network according to the ISO 11898 standard. It presents a typical application circuit and test results, as well as discussions on slope control, total loop delay, and interoperability in 5-V CAN systems.

### 11.1.1   CAN Bus States

The CAN bus has two states during powered operation of the device; dominant and recessive. A dominant bus state is when the bus is driven differentially, corresponding to a logic low on the D and R pin. A recessive bus state is when the bus is biased to VCC / 2 via the high-resistance internal resistors RI  and RDiff  of the receiver, corresponding to a logic high on the D and R pins. See Figure 34 and Figure 35.

▲

1          2          3          4

Typica l Bus Vo ltage (V)

CANH

▲

Vdiff(D)

Vdiff(R)

CANL

Recessive Logic H

Recessive Logic H

Dominant Logic L

Time, t

Figure 34.  CAN Bus States (Physical Bit Representation)

CANH

RXD VCC/2

CANL

Figure 35.  Simplified Recessive Common Mode Bias and Receiver

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018

www.ti.com

## 11.2   Typical Application

Figure 36 illustrates a typical application of the SN65HVD23x family. The output of the host µP's CAN controller (TXD)  is  connected  to  the  transceivers  driver  input,  pin  D,  and  the  transceivers  receiver  output,  pin  R,  is connected to the input of the CAN controller (RXD). The transceiver is attached to the differential bus lines at pins CANH and CANL. Typically, the bus is a twisted pair of wires with a characteristic impedance of 120 Ω , in the standard half-duplex multipoint topology of Figure 37. Each end of the bus is terminated with 120 Ω resistors in compliance with the standard to minimize signal reflections on the bus.

CANTX/IOPC6        CANRX/IOPC7

D                             R

SN65HVD230

CANH                                    CANL

CAN Bus Line

Electronic Control Unit (ECU)

TMS320Lx2403/6/7

CAN-Controller

Figure 36.  Details of a Typical CAN Node

CAN Bus Line

CANL

120 Ω

120 Ω

ECU                                   ECU                                                           ECU

1                                       2                                                              n

CANH

Figure 37.  Typical CAN Network

### 11.2.1   Design Requirements

#### 11.2.1.1   CAN Termination

The ISO11898 standard specifies the interconnect to be a single twisted pair cable (shielded or unshielded) with 120 Ω characteristic impedance (ZO). Resistors equal to the characteristic impedance of the line should be used to terminate both ends of the cable to prevent signal reflections.  Unterminated drop lines (stubs) connecting nodes to the bus should be kept as short as possible to minimize signal reflections. The termination may be on the cable or in a node, but if nodes may be removed from the bus the termination must be carefully placed so that it is not removed from the bus.

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

Typical Application (continued)

Node n

(with termination)

MCU or DSP

CAN

Controller

CAN

Transceiver

RTERM

Node 1

MCU or DSP

CAN

Controller

CAN

Transceiver

Node 2

MCU or DSP

CAN

Controller

CAN

Transceiver

Node 3

MCU or DSP

CAN

Controller

CAN

Transceiver

RTERM

Figure 38.  Typical CAN Bus

Termination is typically a 120 Ω resistor at each end of the bus. If filtering and stabilization of the common mode voltage of the bus is desired, then split termination may be used (see Figure 39). Split termination utilizes two 60Ω  resistors  with  a  capacitor  in  the  middle  of  these  resistors  to  ground.  Split  termination  improves  the electromagnetic emissions behavior of the network by eliminating fluctuations in the bus common mode voltages at the start and end of message transmissions.

Standard Termination

Care should be taken in the power ratings of the termination resistors used. Typically the worst case condition would be if the system power supply was shorted across the termination resistance to ground. In most cases the current flow through the resistor in this condition would be much higher than the transceiver's current limit.

CANH   回                          ●                   m

CAN

Transceiver                  RTERM

CANL

RTERM/2

CSPLIT RTERM/2

CANH

CAN

Transceiver

CANL

Split Termination

Figure 39.  CAN Bus Termination Concepts

#### 11.2.1.2  Loop Propagation Delay

Transceiver loop delay is a measure of the overall device propagation delay, consisting of the delay from the driver input (D pin) to the differential outputs (CANH and CANL pins), plus the delay from the receiver inputs (CANH and CANL) to its output (R pin).

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018

www.ti.com

Typical Application (continued)

A typical loop delay for the SN65HVD230 transceiver is displayed in Figure 40. This loop delay will increase as the slope of the driver output is slowed during slope control mode. This increased loop delay means that there is a tradeoff between the total bus length able to be used and the driver's output slope used via the slope control pin of the device. For example, the loop delay for a 10-kΩ resistor from the RS  pin to ground is ~100 ns, and the loop delay for a 100-kΩ resistor is ~500 ns. Therefore, if we use the following rule-of-thumb that the propagation delay of typical twisted pair bus cable is 5 ns/m, we can calculate an approximate cable length trade-off between normal high-speed mode and slope control mode with a 100-kΩ resistor. Using typical values, the loop delay for a recessive to dominant bit with RS tied directly to ground is 70ns, and with a 100-kΩ resistor is 535 ns. At 5ns/m of propagation delay, which you have to count in both directions the difference is 46.5 meters (535-70)/(2*5).

Another option to improving the elctromagnetic emissions of the device besides slowing down the edge rates of the driver in slope control mode is using quality shielded bus cabling.

(       )

Figure 40.  70.7-ns Loop Delay Through the HVD230 With RS = 0

#### 11.2.1.3  Bus Loading, Length and Number of Nodes

The ISO11898 Standard specifies up to 1 Mbps data rate, maximum bus length of 40 meters, maximum drop line (stub) length of 0.3 meters and a maximum of 30 nodes. However, with careful network design, the system may have longer cables, longer stub lengths, and many more nodes. Many CAN organizations and standards have scaled the use of CAN for applications outside the original ISO11898 standard. They have made system level trade-offs for data rate, cable length, and parasitic loading of the bus. Examples of some of these specifications are ARINC825, CANopen, CAN Kingdom, DeviceNet and NMEA200.

A high number of nodes requires a transceiver with high input impedance and wide common mode range such as the SN65HVD23x CAN family. ISO11898-2 specifies the driver differential output with a 60 Ω load (two 120 Ω termination resistors in parallel) and the differential output must be greater than 1.5 V. The SN65HVD23x devices are specified to meet the 1.5 V requirement with a 60 Ω load, and additionally specified with a differential output voltage minimum of 1.2 V across a common mode range of –2 V to 7 V via a 167 Ω coupling network. This network represents the bus loading of 120 SN65HVD23x transceivers based on their minimum differential input resistance of 40 kΩ . Therefore, the SN65HVD23x supports up to 120 transceivers on a single bus segment with margin to the  1.2 V  minimum  differential  input voltage  requirement at each  node.  For  CAN  network design,

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018 Typical Application (continued)

margin must be given for signal loss across the system and cabling, parasitic loadings, network imbalances, ground offsets and signal integrity thus a practical maximum number of nodes may be lower. Bus length may also be extended beyond the original ISO11898 standard of 40 meters by careful system design and data rate tradeoffs. For example, CANopen network design guidelines allow the network to be up to 1 km with changes in the termination resistance, cabling, less than 64 nodes and significantly lowered data rate.

This flexibility  in  CAN  network  design  is  one  of  the  key  strengths  of  the  various  extensions  and  additional standards  that  have  been  built  on  the  original  ISO11898  CAN  standard.  In  using  this flexibility  comes  the responsibility of good network design.

### 11.2.2   Detailed Design Procedure

The following system level considerations should be looked at when designing your application. There are trade- offs between the total number of nodes, the length of the bus, and the slope of the driver output that need to be evaluated when building up a system

#### 11.2.2.1   Transient Protection

Typical  applications  that   use   CAN  will  sometime   require  some  form  of   ESD,  burst,  or  surge   protection performance at the system level. If these requirements are higher than those of the device some form of external protection may be needed to shield the transceiver against these high power transients that can cause damage. Transient voltage suppressor (TVS) are very commonly used and can help clamp the amount of energy that reaches the transceiver.

#### 11.2.2.2   Transient Voltage Suppressors

Transient voltage suppressors are the preferred protection components for CAN bus applications due to their low capacitance, fast response times and high peak power dissipation limits. The low bus capacitance allows these devices to be used at many, if not all, nodes on the network without having to reduce the data rate. The quick response  times  in  the  order  of  a  few  picoseconds  enable  these  devices  to  clamp  the  energy  of  very  fast transients like ESD and EFT. Lastly, the high peak power ratings enable these devices to handle high energy surge pulses without being damaged.

### 11.2.3   Application Curve

Typical driver output waveforms from a pulse input signal with different slope control resistances are displayed in Figure 41. The top waveform show the typical differential signal when transitioning from a recessive level to a dominant level on the CAN bus with RS tied to GND through a zero ohm resistor. The second waveform shows the same signal for the condition with a 10k ohm resistor tied from RS  to ground. The bottom waveform shows the typical differential signal for the case where a 100k ohm resistor is tied from the RS  pin to ground.

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018

www.ti.com

Typical Application (continued)

RS = 0 Ω

RS = 10 kΩ

RS = 100 kΩ

Figure 41.  Typical SN65HVD230 250-kbps Output Pulse Waveforms With Slope Control

## 11.3   System Example

### 11.3.1   ISO 11898 Compliance of SN65HVD23x Family of 3.3 V CAN Transceivers

#### 11.3.1.1  Introduction

Many users value the low power consumption of operating their CAN transceivers from a 3.3 V supply. However, some are concerned about the  interoperability with 5 V supplied transceivers on the same  bus. This  report analyzes this situation to address those concerns.

#### 11.3.1.2  Differential Signal

CAN  is  a  differential  bus  where  complementary  signals  are sent over two wires  and the voltage difference between the two wires defines the logical state of the bus. The differential CAN receiver monitors this voltage difference and outputs the bus state with a single-ended output signal.

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

System Example (continued)

900

500

mV Threshold

mV Threshold

75% SAMPLE POINT

NOISE MARGIN

RECEIVER DETECTION WINDOW

NOISE MARGIN

Figure 42.  Typical SN65HVD230 Differential Output Voltage Waveform

The CAN driver creates the differential voltage between CANH and CANL in the dominant state. The dominant differential output of the SN65HVD23x is greater than 1.5 V and less than 3 V across a 60 ohm load as defined by the ISO 11898 standard. These are the same limiting values for 5 V supplied CAN transceivers. Typically, the bus termination resistors drive the bus back to the recessive bus state and not the CAN driver.

A CAN receiver is required to output a recessive state when less than 500 mV of differential voltage exists on the bus, and a dominant state when more than 900 mV of differential voltage exists on the bus. The CAN receiver must  do  this  with  common-mode  input  voltages  from  -2  V  to  7  volts  per  the  ISO  11898-2  standard.  The SN65HVD23x family receivers meet these same input specifications as 5 V supplied receivers.

11.3.1.2.1   Common Mode Signal

A common-mode signal is an average voltage of the two signal wires that the differential receiver rejects. The common-mode signal comes from the CAN driver, ground noise, and coupled bus noise. Since the bias voltage of the recessive state of the device is dependent on VCC, any noise present or variation of VCC will have an effect on this bias voltage seen by the bus. The SN65HVD23x family has the recessive bias voltage set higher than 0.5*VCC  to  comply with the ISO  11898-2 CAN standard which states that the recessive bias voltage must be between 2 V and 3 V. The caveat to this  is that the common  mode voltage will drop  by a couple  hundred millivolts when driving a dominant bit on the bus. This means that there is a common mode shift between the dominant  bit and  recessive  bit  states of the device. While this  is  not  ideal, this  small variation  in the  driver common-mode output is rejected by differential receivers and does not effect data, signal noise margins or error rates.

#### 11.3.1.3  Interoperability of 3.3-V CAN in 5-V CAN Systems

The 3.3 V supplied SN65HVD23x family of CAN transceivers are fully compatible with 5 V CAN transceivers. The differential output voltage is the same, the recessive common mode output bias is the same, and the receivers have the same input specifications. The only difference is in the dominant common mode output voltage is lower in 3.3 V CAN transceivers than with 5 V supplied transceiver (by a few hundred millivolts).

To  help  ensure  the  widest  interoperability  possible,  the  SN65HVD23x  family  has  successfully  passed  the internationally  recognized  GIFT  ICT  conformance  and  interoperability  testing  for  CAN  transceivers  which  is shown in . Electrical interoperability does not always assure interchangeability however. Most implementers of CAN buses recognize that ISO 11898 does not sufficiently specify the electrical layer and that strict standard compliance alone does not ensure full interchangeability. This comes only with thorough equipment testing.

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018

www.ti.com

System Example (continued)

One Meter Belden Cable # 3105A

120 Ω                                                                                                                                                                                                                                                   120 Ω

SN65HVD230                              SN65HVD230                              SN65HVD251                           Competitor X251

Trigger Input

TEKTRONIX P6243

Single-Ended Probes

TEKTRONIX

HFS-9003

Pattern

Generator

–    +

HP E3516A

3.3-V Power

Supply

–    +

HP E3516A

5-V Power

Supply

TEKTRONIX 784D

Oscilloscope

Figure 43.  3.3-V and 5-V CAN Transceiver System Testing

# 12   Power Supply Recommendations

The SN65HVD23x 3.3 V CAN transceivers provide the interface between the 3.3 V µPs, MCUs and DSPs and the differential bus lines, and are designed to transmit data at signaling rates up to 1 Mbps as defined by the ISO 11898 standard.

To ensure reliable operation at all data rates and supply voltages, the VCC  supply pin of each CAN transceiver should be decoupled with a 100-nF ceramic capacitor located as close to the VCC  and GND pins as possible. The TPS76333 is a linear voltage regulator suitable for supplying the 3.3-V supply.

SN65HVD230, SN65HVD231, SN65HVD232 www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

# 13   Layout

## 13.1   Layout Guidelines

In order for the PCB design to be successful, start with design of the protection and filtering circuitry. Because ESD and EFT transients have a wide frequency bandwidth from approximately 3 MHz to 3 GHz, high frequency layout techniques must be applied during PCB design. On chip IEC ESD protection is good for laboratory and portable equipment but is usually not sufficient for EFT and surge transients occurring in industrial environments. Therefore robust and reliable bus node design requires the use of external transient protection devices at the bus connectors. Placement at the connector also prevents these harsh transient events from propagating further into the PCB and system.

Use VCC  and  ground planes to provide low inductance. Note: high frequency current follows the path of least inductance and not the path of least resistance.

Design the bus protection components in the direction of the signal path. Do not force the transient current to divert from the signal path to reach the protection device.

An example placement of the Transient Voltage Suppression (TVS) device indicated as D1 (either bi-directional diode or varistor solution) and bus filter capacitors C8 and C9 are shown in .

The bus transient protection and filtering components should be placed as close to the bus connector, J1, as possible. This prevents transients, ESD and noise from penetrating onto the board and disturbing other devices. Bus termination: Figure 44 shows split termination. This is where the termination is split into two resistors, R7 and R8, with the center or split tap of the termination connected to ground via capacitor C7. Split termination provides common mode filtering for the bus. When termination is placed on the board instead of directly on the bus, care must be taken to ensure the terminating node is not removed from the bus as this will cause signal integrity issues of the bus is not properly terminated on both ends. See the application section for information on power ratings needed for the termination resistor(s).

Bypass and bulk capacitors should be placed as close as possible to the supply pins of transceiver, examples C2, C3 (VCC).

Use at least two vias for VCC  and ground connections of bypass capacitors and protection devices to minimize trace and via inductance.

To limit current of digital lines, serial resistors may be used. Examples are R1, R2, R3 and R4.

To filter noise on the digital IO lines, a capacitor may be used close to the input side of the IO as shown by C1 and C4.

Since the internal pull up and pull down biasing of the device is weak for floating pins, an external 1k to 10k ohm pull-up or down resistor should be used to bias the state of the pin more strongly against noise during transient events.

Pin  1:  If  an  open  drain  host  processor  is  used  to  drive the  D  pin  of the  device  an external  pull-up  resistor between 1k and 10k ohms should be used to drive the recessive input state of the device (R1).

Pin 8: is shown assuming the mode pin, RS, will be used. If the device will only be used in normal mode or slope control mode, R3 is not needed and the pads of C4 could be used for the pull down resistor to GND.

Pin 5 in is shown for the SN65HVD230 and SN65HVD231 devices which have a Vref  output voltage reference. If used, this pin should be tied to the common mode point of the split termination. If this feature is not used, the pin can be left floating.

For the  SN65HVD232,  pins  5  and  8  are  no  connect  (NC)  pin.  This  means  that  the  pins  are  not  internally connected and can be left floating.

SN65HVD230, SN65HVD231, SN65HVD232

SLOS346O –MARCH 2001 –REVISED APRIL 2018

www.ti.com

## 13.2   Layout Example

C2

C1

C3

R4

C4

C8

C9

D1

J1

R7

R8

Vref can be routed under the device

RS

U1

VCC                    R1

TXD      R2

GND

VCC

RXD      R3

GND

GND

GND

C7

Figure 44.  SN65HVD23x Board Layout

SN65HVD230, SN65HVD231, SN65HVD232

www.ti.com                                                                                                                                           SLOS346O –MARCH 2001 –REVISED APRIL 2018

# 14   Device and Documentation Support

## 14.1   Related Links

The  table  below  lists  quick  access  links.  Categories  include  technical  documents,  support  and  community resources, tools and software, and quick access to sample or buy.

Table 8. Related Links

| PARTS | PRODUCT FOLDER | SAMPLE & BUY | TECHNICAL DOCUMENTS | TOOLS & SOFTWARE | SUPPORT & COMMUNITY |
| --- | --- | --- | --- | --- | --- |
| SN65HVD230 | Click here | Click here | Click here | Click here | Click here |
| SN65HVD231 | Click here | Click here | Click here | Click here | Click here |
| SN65HVD232 | Click here | Click here | Click here | Click here | Click here |

## 14.2   Receiving Notification of Documentation Updates

To receive notification of documentation updates, navigate to the device product folder on ti.com. In the upper right  corner,  click  on Alert me to  register  and  receive  a  weekly  digest  of  any  product  information  that  has changed. For change details, review the revision history included in any revised document.

## 14.3   Community Resources

The following links connect to TI community resources. Linked contents are provided "AS IS" by the respective contributors. They do not constitute TI specifications and do not necessarily reflect TI's views; see TI's Terms of Use.

TI E2E™ Online Community  TI's Engineer.to.Engineer (E2E) Community. Created to foster collaboration  among engineers. At e2e.ti.com, you can ask questions, share knowledge, explore ideas and help solve problems with fellow engineers.

Design Support TI's Design Support Quickly find helpful E2E forums along with design support tools and contact information for technical support.

## 14.4   Trademarks

E2E is a trademark of Texas Instruments.

All other trademarks are the property of their respective owners.

## 14.5   Electrostatic Discharge Caution

These devices have limited built-in ESD protection. The leads should be shorted together or the device placed in conductive foam during storage or handling to prevent electrostatic damage to the MOS gates.

## 14.6   Glossary

SLYZ022 — TI Glossary.

This glossary lists and explains terms, acronyms, and definitions.

# 15   Mechanical, Packaging, and Orderable Information

The following  pages  include  mechanical,  packaging,  and orderable  information. This  information  is  the  most current data available for the designated devices. This data is subject to change without notice and revision of this document. For browser-based versions of this data sheet, refer to the left-hand navigation.

PACKAGE OPTION ADDENDUM

www.ti.com                                                                                                                                                                                                                                                                                                                                                                   5-Mar-2026

PACKAGING INFORMATION

| Orderable part number | Status (1) | Material type (2) | Package ¦ Pins | Package qty ¦ Carrier | RoHS (3) | Lead finish/ Ball material (4) | MSL rating/ Peak reflow (5) | Op temp (°C) | Part marking (6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SN65HVD230D | Obsolete | Production | SOIC (D) ¦ 8 | - | - | Call TI | Call TI | -40 to 85 | VP230 |
| SN65HVD230DR | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP230 |
| SN65HVD230DR.A | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP230 |
| SN65HVD230DRG4 | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP230 |
| SN65HVD231D | Obsolete | Production | SOIC (D) ¦ 8 | - | - | Call TI | Call TI | -40 to 85 | VP231 |
| SN65HVD231DR | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP231 |
| SN65HVD231DR.A | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP231 |
| SN65HVD231DRG4 | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP231 |
| SN65HVD232D | Obsolete | Production | SOIC (D) ¦ 8 | - | - | Call TI | Call TI | -40 to 85 | VP232 |
| SN65HVD232DR | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP232 |
| SN65HVD232DR.A | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP232 |
| SN65HVD232DRG4 | Active | Production | SOIC (D) ¦ 8 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | VP232 |

(1) Status:  For more details on status, see our product life cycle.

(2) Material type:  When designated, preproduction parts are prototypes/experimental devices, and are not yet approved or released for full production. Testing and final process, including without limitation quality assurance, reliability performance testing, and/or process qualification, may not yet be complete, and this item is subject to further changes or possible discontinuation. If available for ordering, purchases will be subject to an additional waiver at checkout, and are intended for early internal evaluation purposes only. These items are sold without warranties of any kind.

(3) RoHS values:  Yes, No, RoHS Exempt. See the TI RoHS Statement for additional information and value definition.

(4) Lead finish/Ball material:  Parts may have multiple material finish options. Finish options are separated by a vertical ruled line. Lead finish/Ball material values may wrap to two lines if the finish value exceeds the maximum column width.

(5) MSL rating/Peak reflow:  The moisture sensitivity level ratings and peak solder (reflow) temperatures. In the event that a part has multiple moisture sensitivity ratings, only the lowest level per JEDEC standards is shown. Refer to the shipping label for the actual reflow temperature that will be used to mount the part to the printed circuit board.

(6) Part marking:  There may be an additional marking, which relates to the logo, the lot trace code information, or the environmental category of the part.

Multiple part markings will be inside parentheses. Only one part marking contained in parentheses and separated by a "~" will appear on a part. If a line is indented then it is a continuation of the previous line and the two combined represent the entire part marking for that device.

PACKAGE OPTION ADDENDUM

www.ti.com                                                                                                                                                                                                                                                                                                                                                                   5-Mar-2026

Important Information and Disclaimer:The information provided on this page represents TI's knowledge and belief as of the date that it is provided. TI bases its knowledge and belief on information provided by third parties, and makes no representation or warranty as to the accuracy of such information. Efforts are underway to better integrate information from third parties. TI has taken and continues to take reasonable steps to provide representative and accurate information but may not have conducted destructive testing or chemical analysis on incoming materials and chemicals. TI and TI suppliers consider certain information to be proprietary, and thus CAS numbers and other limited information may not be available for release.

In no event shall TI's liability arising out of such information exceed the total purchase price of the TI part(s) at issue in this document sold by TI to Customer on an annual basis.

PACKAGE MATERIALS INFORMATION

www.ti.com                                                                                                                                                                                                                               23-Dec-2025

TAPE AND REEL INFORMATION

TAPE DIMENSIONS

K0         P1

W B0

Reel

Diameter

Cavity           A0

| A0 | Dimension designed to accommodate the component width |
| --- | --- |
| B0 | Dimension designed to accommodate the component length |
| K0 | Dimension designed to accommodate the component thickness |
| W | Overall width of the carrier tape |
| P1 | Pitch between successive cavity centers |

REEL DIMENSIONS

Reel Width (W1)

QUADRANT ASSIGNMENTS FOR PIN 1 ORIENTATION IN TAPE

4— Sprocket Holes

User Direction of Feed

Q1

Q3

Q2

Q4

Q1

Q3

Q2

Q4

Pocket Quadrants

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Reel Diameter (mm) | Reel Width W1 (mm) | A0 (mm) | B0 (mm) | K0 (mm) | P1 (mm) | W (mm) | Pin1 Quadrant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SN65HVD230DR | SOIC | D | 8 | 2500 | 330.0 | 12.4 | 6.4 | 5.2 | 2.1 | 8.0 | 12.0 | Q1 |
| SN65HVD231DR | SOIC | D | 8 | 2500 | 330.0 | 12.4 | 6.4 | 5.2 | 2.1 | 8.0 | 12.0 | Q1 |
| SN65HVD232DR | SOIC | D | 8 | 2500 | 330.0 | 12.4 | 6.4 | 5.2 | 2.1 | 8.0 | 12.0 | Q1 |

www.ti.com                                                                                                                                                                                                                               23-Dec-2025

TAPE AND REEL BOX DIMENSIONS

H

L

W

可

A

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Length (mm) | Width (mm) | Height (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SN65HVD230DR | SOIC | D | 8 | 2500 | 353.0 | 353.0 | 32.0 |
| SN65HVD231DR | SOIC | D | 8 | 2500 | 353.0 | 353.0 | 32.0 |
| SN65HVD232DR | SOIC | D | 8 | 2500 | 353.0 | 353.0 | 32.0 |

D0008A

PACKAGE OUTLINE SOIC - 1.75 mm max height

SMALL OUTLINE INTEGRATED CIRCUIT

0 - 8 .016-.050 [0.41-1.27] .004-.010 [0.11-0.25] DETAIL A .005-.010 TYP [0.13-0.25] 4X (0 -15 ) SEE DETAIL A .010 [0.25] .228-.244 TYP [5.80-6.19] PIN 1 ID AREA 6X .050 [1.27] 1 8 A .189-.197 [4.81-5.00] NOTE 3 C SEATING PLANE .004 [0.1] C 5 8X .012-.020 [0.31-0.51] .010 [0.25] .150-.157 [3.81-3.98] NOTE 4 .069 MAX [1.75] 4 B 2X .150 [3.81] 4X (0 -15 ) C A B (.041) TYPICAL [1.04] 4214825/C 02/2019

NOTES:

1. Linear dimensions are in inches [millimeters]. Dimensions in parenthesis are for reference only. Controlling dimensions are in inches. Dimensioning and tolerancing per ASME Y14.5M.

2. This drawing is subject to change without notice.

3. This dimension does not include mold flash, protrusions, or gate burrs. Mold flash, protrusions, or gate burrs shall not exceed .006 [0. 15] per side.

4. This dimension does not include interlead flash.

5. Reference JEDEC registration MS-012, variation AA.

EXAMPLE BOARD LAYOUT D0008A                                                           SOIC - 1.75 mm max height

SMALL OUTLINE INTEGRATED CIRCUIT

| 4 SEE DETAILS 8 (R.002 ) TYP [0.05] SYMM 6X (.050 ) [1.27] 8X (.061 ) [1.55] 8X (.024) [0.6] (.213) [5.4] SYMM 5 1 LAND PATTERN EXAMPLE EXPOSED METAL SHOWN SCALE:8X |  |
| --- | --- |
| SOLDER MASK OPENING EXPOSED METAL METAL .0028 MAX [0.07] ALL AROUND NON SOLDER MASK DEFINED | .0028 MIN [0.07] EXPOSED METAL SOLDER MASK OPENING METAL UNDER SOLDER MASK ALL AROUND SOLDER MASK DEFINED |
| SOLDER MASK DETAILS 4214825/C 02/2019 |  |

NOTES: (continued)

6. Publication IPC-7351 may have alternate designs.

7. Solder mask tolerances between and around signal pads can vary based on board fabrication site.

EXAMPLE STENCIL DESIGN D0008A                                                           SOIC - 1.75 mm max height

SMALL OUTLINE INTEGRATED CIRCUIT

(R.002 ) TYP [0.05] 6X (.050 ) [1.27] 8X (.061 ) [1.55] 8X (.024) [0.6] (.213) [5.4] SYMM SYMM 8 5 4 1 SOLDER PASTE EXAMPLE BASED ON .005 INCH [0.125 MM] THICK STENCIL SCALE:8X 4214825/C 02/2019

NOTES: (continued)

8. Laser cutting apertures with trapezoidal walls and rounded corners may offer better paste release. IPC-7525 may have alternate design recommendations.

9. Board assembly site may have different recommendations for stencil design.

IMPORTANT NOTICE AND DISCLAIMER

TI PROVIDES TECHNICAL AND RELIABILITY DATA (INCLUDING DATASHEETS), DESIGN RESOURCES (INCLUDING REFERENCE DESIGNS), APPLICATION OR OTHER DESIGN ADVICE, WEB TOOLS, SAFETY INFORMATION, AND OTHER RESOURCES “AS IS” AND WITH ALL FAULTS, AND DISCLAIMS ALL WARRANTIES, EXPRESS AND IMPLIED, INCLUDING WITHOUT LIMITATION ANY

IMPLIED WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE OR NON-INFRINGEMENT OF THIRD PARTY INTELLECTUAL PROPERTY RIGHTS.

These resources are intended for skilled developers designing with TI products. You are solely responsible for (1) selecting the appropriate TI products for your application, (2) designing, validating and testing your application, and (3) ensuring your application meets applicable  standards, and any other safety, security, regulatory or other requirements.

These resources are subject to change without notice. TI grants you permission to use these resources only for development of an

application that uses the TI products described in the resource. Other reproduction and display of these resources is prohibited. No license  is granted to any other TI intellectual property right or to any third party intellectual property right. TI disclaims responsibility for, and you fully indemnify TI and its representatives against any claims, damages, costs, losses, and liabilities arising out of your use of these resources.

TI’s products are provided subject to TI’s Terms of Sale, TI’s General Quality Guidelines, or other applicable terms available either on

ti.com or provided in conjunction with such TI products. TI’s provision of these resources does not expand or otherwise alter TI’s applicable warranties or warranty disclaimers for TI products. Unless TI explicitly designates a product as custom or customer-specified, TI products are standard, catalog, general purpose devices.

TI objects to and rejects any additional or different terms you may propose.

Copyright © 2026, Texas Instruments Incorporated

Last updated 10/2025